Cisco Static Routing, DHCP and DNS Network Project

Network Design, Device Configuration and Protocol Documentation

Mostafa GUELLIL

# Cisco Static Routing DHCP and DNS Practice Lab

Project Overview Protocol Reference and Configuration Record

This document records the three-site Cisco Packet Tracer project that I completed. It preserves the topology, addressing plan, protocol explanations, device configurations, and verification commands so I can return to the project later and understand exactly how it was built.

I connected three routers in a triangle and assigned one LAN to each router. Each router was configured as the default gateway and DHCP server for its local PCs. I placed a DNS server at Site 2 and configured primary static routes together with floating backup routes for WAN link failure.

| Item | Value |
| --- | --- |
| Routers | R1 R2 and R3 using Cisco 2911 |
| Switches | SW1 SW2 and SW3 using Cisco 2960 |
| Clients | Six PCs using DHCP |
| Server | DNS server at 192.168.20.10 |
| LANs | 192.168.10.0 192.168.20.0 and 192.168.30.0 slash 24 |
| WAN design | Three point to point slash 30 networks in a triangle |

# 1 Lab Overview

I built the project as three small office sites. R1, R2, and R3 route traffic between the sites, while SW1, SW2, and SW3 provide local Ethernet access. I placed two PCs at each site and added the DNS server to Site 2. The router at each site was configured as the default gateway and DHCP server for that LAN.

## Project Scope

I assigned IPv4 addresses, subnet masks, and default gateways to the routed interfaces.

I enabled the router interfaces with the no shutdown command.

I created DHCP pools that supplied addressing, gateway, DNS, and domain information.

I configured DNS A records for the main infrastructure devices.

I added primary static routes and floating backup static routes.

I recorded switch management addresses, access ports, and PortFast settings.

I retained verification and troubleshooting commands for future reference.

## Logical Topology

WAN triangle

R1 to R2 uses 10.0.12.0 slash 30

R1 to R3 uses 10.0.13.0 slash 30

R2 to R3 uses 10.0.23.0 slash 30

## Cabling

I used copper straight-through cables between unlike Ethernet devices, including PC-to-switch, server-to-switch, and router-to-switch links. I used copper crossover cables between the router Ethernet interfaces.

| Source | Destination | Cable |
| --- | --- | --- |
| PC1 FastEthernet0 | SW1 FastEthernet0 1 | Straight through |
| PC2 FastEthernet0 | SW1 FastEthernet0 2 | Straight through |
| SW1 GigabitEthernet0 1 | R1 GigabitEthernet0 0 | Straight through |
| PC3 FastEthernet0 | SW2 FastEthernet0 1 | Straight through |
| PC4 FastEthernet0 | SW2 FastEthernet0 2 | Straight through |
| DNS Server FastEthernet0 | SW2 FastEthernet0 3 | Straight through |
| SW2 GigabitEthernet0 1 | R2 GigabitEthernet0 0 | Straight through |
| PC5 FastEthernet0 | SW3 FastEthernet0 1 | Straight through |
| PC6 FastEthernet0 | SW3 FastEthernet0 2 | Straight through |
| SW3 GigabitEthernet0 1 | R3 GigabitEthernet0 0 | Straight through |
| R1 | R2 | Crossover |
| R1 | R3 | Crossover |
| R2 | R3 | Crossover |

# 2 Addressing Plan

I assigned a slash 24 mask to each LAN, providing up to 254 usable host addresses. I assigned a slash 30 mask to every point-to-point WAN, providing exactly two usable endpoint addresses without wasting address space.

| Device | Interface | IPv4 Address | Mask | Purpose |
| --- | --- | --- | --- | --- |
| R1 | G0 0 | 192.168.10.1 | 255.255.255.0 | Site 1 gateway |
| R1 | G0 1 | 10.0.12.1 | 255.255.255.252 | R1 to R2 |
| R1 | G0 2 | 10.0.13.1 | 255.255.255.252 | R1 to R3 |
| R2 | G0 0 | 192.168.20.1 | 255.255.255.0 | Site 2 gateway |
| R2 | G0 1 | 10.0.12.2 | 255.255.255.252 | R2 to R1 |
| R2 | G0 2 | 10.0.23.1 | 255.255.255.252 | R2 to R3 |
| R3 | G0 0 | 192.168.30.1 | 255.255.255.0 | Site 3 gateway |
| R3 | G0 1 | 10.0.23.2 | 255.255.255.252 | R3 to R2 |
| R3 | G0 2 | 10.0.13.2 | 255.255.255.252 | R3 to R1 |
| SW1 | VLAN 1 | 192.168.10.2 | 255.255.255.0 | Management |
| SW2 | VLAN 1 | 192.168.20.2 | 255.255.255.0 | Management |
| SW3 | VLAN 1 | 192.168.30.2 | 255.255.255.0 | Management |
| DNS Server | FastEthernet0 | 192.168.20.10 | 255.255.255.0 | DNS service |

## DHCP Address Ranges

| Site | Network | Excluded Addresses | Client Range | Gateway |
| --- | --- | --- | --- | --- |
| Site 1 | 192.168.10.0 slash 24 | 192.168.10.1 to 20 | 192.168.10.21 to 254 | 192.168.10.1 |
| Site 2 | 192.168.20.0 slash 24 | 192.168.20.1 to 20 | 192.168.20.21 to 254 | 192.168.20.1 |
| Site 3 | 192.168.30.0 slash 24 | 192.168.30.1 to 20 | 192.168.30.21 to 254 | 192.168.30.1 |

# 3 Protocols and Networking Concepts

## Ethernet

Ethernet is the local network technology used by the PCs, server, switches, and router LAN interfaces. It defines how frames are carried across copper links and identifies interfaces with MAC addresses.

## IPv4

Internet Protocol version 4 provides logical addressing and allows routers to forward packets between networks. Every routed interface in this project has an IPv4 address and subnet mask.

## Subnet Mask and CIDR

A subnet mask separates the network part of an address from the host part. The LAN mask 255.255.255.0 is written as slash 24. The WAN mask 255.255.255.252 is written as slash 30 and supports two usable endpoint addresses.

## ARP

Address Resolution Protocol maps a local IPv4 address to a MAC address. Before a PC can send a frame to its gateway or another local host, it uses ARP to learn the destination MAC address. The first ping may time out while ARP completes.

## ICMP

Internet Control Message Protocol carries diagnostic and error messages. The ping command uses ICMP Echo Request and Echo Reply messages to test reachability.

## DHCP

Dynamic Host Configuration Protocol automatically supplies a client with an IPv4 address, subnet mask, default gateway, DNS server, and domain name. Clients use UDP port 68 and servers use UDP port 67. The normal exchange is Discover Offer Request Acknowledge, often called DORA.

## DNS

Domain Name System translates names into IP addresses. This lab uses DNS A records, which map IPv4 host names such as dns.practice.lab to 192.168.20.10. DNS normally uses UDP port 53 and can use TCP port 53 for larger responses or zone transfers.

## Static Routing

A static route is entered manually by an administrator. It tells a router which next-hop address to use for a destination network. Static routing is predictable and useful in small labs, but it does not automatically discover networks like a dynamic routing protocol.

## Floating Static Route

A floating static route is a backup route with a higher administrative distance than the preferred route. The backup routes in this lab use administrative distance 10, so they enter the routing table only when the primary route is unavailable.

## Administrative Distance

Administrative distance measures how trustworthy a route source is. A normal static route has administrative distance 1 by default. A static route configured with distance 10 is less preferred and therefore acts as a backup.

## Default Gateway

The default gateway is the router address that a host uses when the destination is outside its local subnet. Each PC receives its local router LAN address as the gateway through DHCP.

## VLAN 1

A VLAN creates a logical Layer 2 broadcast domain. This simple lab uses the default VLAN 1. Each switch receives a VLAN 1 management address so it can be reached from the network.

## Spanning Tree Protocol

STP prevents Layer 2 loops by controlling redundant switch paths. The topology has no redundant switch links, but STP still runs on the Cisco switches.

## PortFast

PortFast allows an access port connected to an endpoint to move to the forwarding state quickly. It should be used only on endpoint-facing ports, not between switches.

## TCP and UDP

TCP and UDP are transport protocols. TCP provides connection-oriented reliable delivery. UDP has less overhead and supports time-sensitive request and response traffic. DHCP uses UDP, while DNS can use either UDP or TCP.

# 4 Router Configuration

I configured all three routers with hostnames, IPv4 addresses, DHCP service, primary static routes, and floating backup routes. I used no ip domain lookup to prevent mistyped commands from causing DNS lookup delays. I added interface descriptions for reference and used no shutdown to enable each interface.

## R1 Configuration

I configured R1 as the default gateway and DHCP server for Site 1. I set its preferred path to Site 2 through the direct R1-to-R2 link and its preferred path to Site 3 through the direct R1-to-R3 link.

enable

configure terminal

hostname R1

no ip domain-lookup

interface gigabitEthernet0/0

description LAN_TO_SW1

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet0/1

description WAN_TO_R2

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet0/2

description WAN_TO_R3

ip address 10.0.13.1 255.255.255.252

no shutdown

exit

ip dhcp excluded-address 192.168.10.1 192.168.10.20

ip dhcp pool SITE1_LAN

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 192.168.20.10

domain-name practice.lab

exit

ip route 192.168.20.0 255.255.255.0 10.0.12.2

ip route 192.168.20.0 255.255.255.0 10.0.13.2 10

ip route 192.168.30.0 255.255.255.0 10.0.13.2

ip route 192.168.30.0 255.255.255.0 10.0.12.2 10

end

copy running-config startup-config

## R2 Configuration

I configured R2 as the default gateway and DHCP server for Site 2. It also provides routed access to the DNS server on the Site 2 LAN.

enable

configure terminal

hostname R2

no ip domain-lookup

interface gigabitEthernet0/0

description LAN_TO_SW2

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet0/1

description WAN_TO_R1

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet0/2

description WAN_TO_R3

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

ip dhcp excluded-address 192.168.20.1 192.168.20.20

ip dhcp pool SITE2_LAN

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 192.168.20.10

domain-name practice.lab

exit

ip route 192.168.10.0 255.255.255.0 10.0.12.1

ip route 192.168.10.0 255.255.255.0 10.0.23.2 10

ip route 192.168.30.0 255.255.255.0 10.0.23.2

ip route 192.168.30.0 255.255.255.0 10.0.12.1 10

end

copy running-config startup-config

## R3 Configuration

I configured R3 as the default gateway and DHCP server for Site 3. Its backup routes use the remaining router when a direct WAN path is unavailable.

enable

configure terminal

hostname R3

no ip domain-lookup

interface gigabitEthernet0/0

description LAN_TO_SW3

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet0/1

description WAN_TO_R2

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet0/2

description WAN_TO_R1

ip address 10.0.13.2 255.255.255.252

no shutdown

exit

ip dhcp excluded-address 192.168.30.1 192.168.30.20

ip dhcp pool SITE3_LAN

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 192.168.20.10

domain-name practice.lab

exit

ip route 192.168.10.0 255.255.255.0 10.0.13.1

ip route 192.168.10.0 255.255.255.0 10.0.23.1 10

ip route 192.168.20.0 255.255.255.0 10.0.23.1

ip route 192.168.20.0 255.255.255.0 10.0.13.1 10

end

copy running-config startup-config

# 5 Switch Configuration

I configured the switches to operate at Layer 2 and assigned a VLAN 1 address to each switch for management. I set the management default gateway with ip default gateway and configured the endpoint ports in access mode with PortFast.

## SW1 Configuration

I used SW1 to connect the two Site 1 PCs to R1 and assigned 192.168.10.2 as its management address.

enable

configure terminal

hostname SW1

no ip domain-lookup

interface vlan 1

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.10.1

interface range fastEthernet0/1-2

description SITE1_PCS

switchport mode access

spanning-tree portfast

exit

interface gigabitEthernet0/1

description UPLINK_TO_R1

switchport mode access

exit

end

copy running-config startup-config

## SW2 Configuration

I used SW2 to connect the two Site 2 PCs and the DNS server to R2 and assigned 192.168.20.2 as its management address.

enable

configure terminal

hostname SW2

no ip domain-lookup

interface vlan 1

ip address 192.168.20.2 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.20.1

interface range fastEthernet0/1-3

description SITE2_HOSTS

switchport mode access

spanning-tree portfast

exit

interface gigabitEthernet0/1

description UPLINK_TO_R2

switchport mode access

exit

end

copy running-config startup-config

## SW3 Configuration

I used SW3 to connect the two Site 3 PCs to R3 and assigned 192.168.30.2 as its management address.

enable

configure terminal

hostname SW3

no ip domain-lookup

interface vlan 1

ip address 192.168.30.2 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.30.1

interface range fastEthernet0/1-2

description SITE3_PCS

switchport mode access

spanning-tree portfast

exit

interface gigabitEthernet0/1

description UPLINK_TO_R3

switchport mode access

exit

end

copy running-config startup-config

# 6 PC Configuration Record

I configured all six PCs as DHCP clients from Desktop and IP Configuration. The exact client address depends on the order in which the PCs request leases. Each client was assigned to its local site range and was set to receive the DNS server address 192.168.20.10 through DHCP.

| PCs | Expected Network | Expected Gateway | DNS Server |
| --- | --- | --- | --- |
| PC1 and PC2 | 192.168.10.0 slash 24 | 192.168.10.1 | 192.168.20.10 |
| PC3 and PC4 | 192.168.20.0 slash 24 | 192.168.20.1 | 192.168.20.10 |
| PC5 and PC6 | 192.168.30.0 slash 24 | 192.168.30.1 | 192.168.20.10 |

# 7 DNS Server Configuration Record

I assigned the DNS server a fixed address so every DHCP client could use a stable DNS destination. Because this server hosts the local DNS service, I also entered its own address as the DNS server value.

## Static IPv4 Settings

| Setting | Configured Value |
| --- | --- |
| IPv4 address | 192.168.20.10 |
| Subnet mask | 255.255.255.0 |
| Default gateway | 192.168.20.1 |
| DNS server | 192.168.20.10 |

## DNS Service and Records

I enabled the DNS service and recorded the following IPv4 A records for the infrastructure devices.

| DNS Name | Record Type | IPv4 Address |
| --- | --- | --- |
| dns.practice.lab | A | 192.168.20.10 |
| r1.practice.lab | A | 192.168.10.1 |
| r2.practice.lab | A | 192.168.20.1 |
| r3.practice.lab | A | 192.168.30.1 |
| sw1.practice.lab | A | 192.168.10.2 |
| sw2.practice.lab | A | 192.168.20.2 |
| sw3.practice.lab | A | 192.168.30.2 |

I did not make the PC names permanent DNS records because DHCP addresses can change. The current leases can be checked later with show ip dhcp binding on each router.

# 8 How Static Routing Works in This Lab

Each router already knows its directly connected LAN and WAN networks. I therefore added static routes only for the two remote LANs. For every remote LAN, I recorded a normal primary route and a floating route with administrative distance 10.

| Router | Destination | Primary Next Hop | Backup Next Hop |
| --- | --- | --- | --- |
| R1 | 192.168.20.0 slash 24 | 10.0.12.2 | 10.0.13.2 |
| R1 | 192.168.30.0 slash 24 | 10.0.13.2 | 10.0.12.2 |
| R2 | 192.168.10.0 slash 24 | 10.0.12.1 | 10.0.23.2 |
| R2 | 192.168.30.0 slash 24 | 10.0.23.2 | 10.0.12.1 |
| R3 | 192.168.10.0 slash 24 | 10.0.13.1 | 10.0.23.1 |
| R3 | 192.168.20.0 slash 24 | 10.0.23.1 | 10.0.13.1 |

For example, R1 normally reaches Site 2 directly through R2 at 10.0.12.2. If that link becomes unavailable, the design allows R1 to install the route through R3 at 10.0.13.2, after which R3 forwards the traffic to R2.

# 9 Verification Reference

## Router Commands Retained for Reference

show ip interface brief

show ip route

show ip route static

show ip dhcp pool

show ip dhcp binding

show running-config

I retained these commands because show ip interface brief displays interface addresses and status, show ip route displays connected and static routes, and show ip dhcp binding displays current DHCP leases.

## PC Commands Retained for Reference

ipconfig /all

ping 192.168.20.10

ping 192.168.30.1

ping dns.practice.lab

tracert 192.168.30.21

I recorded the checks in the order of local gateway, remote router, DNS server by address, and DNS name. This order makes it easier to separate local addressing problems from routing and DNS problems when I return to the project.

## Floating Route Failure Design

The normal R1 path to Site 2 uses next hop 10.0.12.2. I configured the floating route so that shutting the direct R1-to-R2 interface makes the route through R3 at 10.0.13.2 eligible for installation. After the direct link is restored with no shutdown, the lower-distance primary route becomes preferred again.

# 10 Troubleshooting

| Symptom | Likely Cause | Check or Fix |
| --- | --- | --- |
| Interface is administratively down | The interface has not been enabled | Enter interface configuration mode and use no shutdown |
| PC has 0.0.0.0 or 169.254 address | DHCP request failed | Check cable status router LAN interface and DHCP pool |
| Local gateway responds but remote LAN does not | Static route or WAN addressing problem | Check show ip route and both ends of each slash 30 link |
| IP ping works but name ping fails | DNS address service or record is wrong | Check client DNS 192.168.20.10 and the server A record |
| Switch management address is unreachable | VLAN 1 or default gateway is wrong | Check interface vlan 1 and ip default-gateway |
| Backup route does not appear | Primary route is still active or backup distance is missing | Shut the primary link and check that the backup route uses distance 10 |
| First ping fails but later pings work | ARP resolution is occurring | Repeat the ping after the ARP table is populated |

# 11 Configuration Summary

I configured R1, R2, and R3 to provide routing and DHCP. I used SW1, SW2, and SW3 for Ethernet access and assigned each switch a management address. The six PCs were set as DHCP clients. The DNS server was assigned the fixed address 192.168.20.10 and the practice.lab records. Primary static routes define the preferred paths, while floating static routes provide the recorded backup paths for a single WAN link failure.